# MENG 356: Mechanical Design I

Problem sheet 3 (Manual Transmission)

Steps for drawing the shaft with splines cross-section:

- Draw the two circles with inner and outer diameter

- Draw construction lines (using lines or polylines command) to indicate the position of each tooth. The angle between each two lines is 60deg.

- Draw one tooth at one of the construction lines and then copy it to the other lines through the following:

  - Draw a horizontal line with half the thickness of the tooth at the intersection of the construction line with the outer circle.

  - From the end of the drawn line draw another line to intersect downwards with the inner circle. Now a half tooth is drawn.

  - Do the same thing for the other side of the tooth to draw the complete tooth.

  - Use command “rotate” with copy sw on to rotate each tooth an angle 60deg to the next construction line.

- When done with the six teeth, delete the two circles.

- Connect each tooth by the next using the command “arc with a center, start, end”.

- When done with connecting the teeth, delete the construction lines and build the section to have it as one part

- Make two other copies of it using the command “move with copy sw on”

- Extrude the original along the shaft length to get the complete shape of the shaft.

- Draw the gears using the command “revolve” for one and the command “extrude” for the other.

- Use a copy of the drawn shaft with a spline cross-section to cut out (under the command extrude) through the gears to form the inner shape of the gears.

Eng. Hossam Gharib